©2007 RealTEK Semiconductor Corp. All rights reserved. No part of this document may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language in any form or by any means without the written permission of RealTEK Semiconductor Corp.

RTL8188E Mass Production UI Guideline

# Document Number:  01-001-G1-0001

Naming Rule: Reference Internal Engineer Development Document Code

Example: 01 0001 H1 01

Serial Number

RTL8190

Wireless NIC

High Level Design Firmware Architecture

Document Code Management

|  |  |  | Code |
| --- | --- | --- | --- |
| RTL Document |  | Functional Spec | F1 |
|  |  | High Level Design | H1 |
|  |  | Low Level Design | L1 |
|  |  | Test Plan, Test Report | T1 |
|  |  | Guideline | G1 |
|  |  | Meeting Notes, Decisions | M1 |

Realtek Semiconductor Corp.

Headquarters

1F, No. 2, Industry East Road IX, Science-based

Industrial Park, Hsinchu, 300, Taiwan, R.O.C.

Tel: 886-3-5780211  Fax: 886-3-5776047

http://www.realtek.com.tw/

# Change History

| Version | Editor | Date | Remarks |
| --- | --- | --- | --- |
| 0.01 | MHC | 08/09/2007 08/13/2007 | - MP guide initial version. - Move content from MPUsage.doc (Cosa). - Cut picture for different setting and display block on MP UI. |
| 0.02 | MHC | 12/15/2008 | - Add 8192SE/SU series MP UI description. - Add new WPG description. |
| 0.03 | MHC | 01/14/2009 | - Modify 92S MP UI figure and document arch. - Modify Efuse opreation flow. - Add tx power tracking switch button. |
| 0.04 | chiyoko | 08/19/2011 | - For 8723A |
| 0.05 | chiyoko | 11/10/2011 | - For 8188E |

“你可以開啟word  檢視  文件導引模式來幫助你閱讀此份文件”

# Table of Contents:

Document Number:  01-001-G1-0001	2

Change History	3

Table of Contents:	4

List of Figure	5

List of Table	6

1.	MP UI Interface	7

2.	Setting Block	8

2.1.	Testing Item	8

2.2.	TX Power	8

2.3.	Channel	8

2.4.	Data Rate	9

2.5.	Preamble	9

2.6.	Signal Location	9

2.7.	ANT TX/RX	9

2.8.	ANT Gain Offset	9

3.	TX Packet Setting Block	10

4.	Mac Address Setting Block	11

5.	RX Filter Setting Block	12

5.1.	Switch Restore RCR / Rx Physical Match only	12

6.	MAC/BB/RF Register/PowerTrack Block	13

6.1.	MAC/BB/RF Register Operate	13

6.2.	Tx Power Tracking Switch	13

7.	EEPROM/Efuse Setting Block	14

7.1.	EEPROM/EFUSE Display Field	14

7.2.	EEPROM Operation Flow	15

7.3.	Efuse Operation Flow	15

8.	HW Setting Map and RF Info	16

9.	TX Result Display Block	17

10.	Other	18

# List of Figure

Figure 1.	MP UI	7

Figure 2.	MP UI  Setting Block	8

Figure 3.	MP UI  TX Packet Setting Block	10

Figure 4.	MP UI  MAC Address Setting Block	11

Figure 5.	MP UI  RX Filter Setting Block	12

Figure 6.	MP UI  MAC/RF Register/PowerTrack Block	13

Figure 7.	MP UI  EEPROM/EFUSE Setting Block	14

Figure 8.	Test & Result Display Block	17

Figure 9.	LED ON/OFF Button	18

# List of Table

Table 1.	Visible Antenna for Different Rate	9

# MP UI Interface

When you press “REALTEK 8188E PCIE/USB WLAN NIC Massproduction Kit” icon, MP UI will be opened for your test and testing block is simply figured as below. We separate as several control blocks. (1). Setting Block. (2). TX Packet Setting Block. (3). MAC Address Block. (4). RX Filter Block. (5).MAC/RF Register/PowerTrack Block. (6). EEPROM/EFUSE Block. (7). Test Result Display Block. We will describe the control blocks chapter by chapter. On MP UI, we only support several action items. (1). Start Testing  Enable operation after you finish the setting of control blocks. (2). Reset  Clear all content in TX Result Block. (3). Quit  Exit MP UI. (Or you can press “ESC” button to leave MP UI quickly).

- MP UI

# Setting Block

- MP UI  Setting Block

## Testing Item

- Packets TX: To transmit packets.

- Packets Rx: To receive packets.

- Continuous TX: To do CCK and OFDM/MCS continuous TX.

- Single Tone Tx: To do OFDM Single tone TX.

- Carrier Suppression: To do CCK Carrier suppression.

- WPS Button: To polling if WPS Button is pushed or not, result will be displayed on Test Result block. (Note: This feature is only apply for USB adapter).

## TX Power

Choose the TX power index. Provide 0~63 power index.

## Channel

Choose the channel you want to transmit packets. Provide channel 1~14.

## Data Rate

- CCK : 1M, 2M, 5.5M, 11M

- OFDM : 6M, 9M, 12M, 18M, 24M, 36M, 48M, 54M

- MCS : 0~7 for 1T solution, 0~15 for 2T solution.

## Preamble

- When Data Rate is CCK rate:

Only Long/Short Preamble makes sense.

- When Data Rate is OFDM rate:

The preamble doesn’t make sense.

- When Data Rate is MCS rate:

Only Long/Short GI makes sense.

## Signal Location

You can choose 20M or 40M bandwidth.

## ANT TX/RX

Select different antenna TX/RX path. Several combinations of the TX/RX RF are shown as the following table.

- Visible Antenna for Different Rate

| Rate\ Antenna | TX | RX |
| --- | --- | --- |
| CCK | A or B | A or B |
| Legacy OFDM, MCS0~7 | A or B | A or B or AB |
| MCS 8~15 | AB | AB |

# TX Packet Setting Block

- MP UI  TX Packet Setting Block

- Packet Pattern: Choose the data pattern you want when transmit packets. You can choose random or some fixed data patterns.

- Packet Length: Before transmitting packets, you can set the packet length here. Range is from 64~1600.

- Packet Counts: Input a number before you start Packets TX testing. Once the transmitted packets reach the number, then MP stops Packets TX testing.

# Mac Address Setting Block

- MP UI  MAC Address Setting Block

You can get adapter’s MAC address here and set the destination Mac address.

- Self Get: Press the button to get adapter’s MAC address.

- Tx Dest Set: For unicast packet of MP, you can input the destination MAC address and press the button to do it. For example, you can input 112233445566 and press the button, and then all packets will be sent to detination with the MAC address.

# RX Filter Setting Block

- MP UI  RX Filter Setting Block

When doing Packets Rx testing, this block can help you to filter received packets. You can force MP to receive the packet what you want.

## Switch Restore RCR / Rx Physical Match only

When press the [Rx Physical Match only] button, then adapter only receive the packets with destination MAC address equal to that of the adapter. After that, the button will show [Restore RCR]. When you press [Restore RCR], the adapter receives all the packets, not only the matched MAC address packets, and the button will become [Rx Physical Match only] again.

# MAC/BB/RF Register/PowerTrack Block

- MP UI  MAC/RF Register/PowerTrack Block

## MAC/BB/RF Register Operate

In the block, you can read or write dedicated MAC/RF register. Please refer the related data sheet when you use the setting block. Otherwise, you may type incorrect register address and system will be crashed. There are several items in the block which we will explain as below.

- RF: When you want to read or write RF register, you must select dedicated RF to work. Now, we support RF A to B.

- Offset: MAC/RF register address. This is a HEX value. If you input A00, it means that you want to read or write MAC register 0xA00.

- Value: When you press REG/RF Read /Thermal Val/ TSSI Val button, the content of dedicated register address will be displayed in the column. On the other hand, the value in the column will be inserted into dedicate register address if you press write button. This is a HEX value.

- REG Read/ REG Write: Before you press read button, you must make sure that MAC register offset is legal. Before you press write button, you have to issue a legal register offset and value.

- RF Read/ RF Write: Before you press read button, you must make sure that RF path and register offset is legal. Before you press write button, you have to issue a legal register offset and value for dedicated RF path.

## Tx Power Tracking Switch

In initial step, tx power tracking scheme is stopped. When you want to enable tx power tracking, please press “TxPwrTrack Start” button.Then NIC will execute tx power tracking and button become to display “TxPwrTrack Start”. You can press the button to stop tx power tracking.

# EEPROM/Efuse Setting Block

- MP UI  EEPROM/EFUSE Setting Block

## EEPROM/EFUSE Display Field

In RTL8192S, we support 128 bytes EEPROM or Efuse map files now. You can read or modify EEPROM/Efuse content by the control block. There are several items in the block which will be explained as below.

- Offset: When you press [EE via Byte], you have to input the offset to read/write EEPROM/EFUSE.

- Value: When you press [EE via Byte], you have to input the value to write EEPROM/EFUSE.

- EE via File/ EE via Byte: (1).After pressing [EE via Byte] button, you can read EEPROM/EFUSE by issuing the Offset or write EEPROM/EFUSE by issuing Offset and Value. At the moment, the button stays in [EE via File] (2).After pressing [EE via File], Offset and Value column do not make sense.

- [R Hw Map] /[W Hw Map]:[R Hw Map] button will display the EFUSE content(shadow table in our MP driver). [W Hw Map] button will transfer the content in file(Ex: 819xse.map) into EFUSE(shadow table in our MP driver). (You can modify the file under MP working directory.)

- [U EFuse]: Push this button will really program the EFUSE content(shadow table in out MP driver)to the real EFuse area.

- Efuse Used: Efuse used bytes. 92S series support 512 bytes Efuse area.

## EEPROM Operation Flow

For EEPROM operation, when you press “R Hw Map” button, you can read EEPROM from NIC. When you press “W Hw Map”, you can write the content from map file into all EEPROM area. (93C46:128 bytes, 93C56: 256 bytes).

## Efuse Operation Flow

For EFUSE operation, when you press “W Hw Map” button, you only modify a shadow table in our MP driver. If you really want to write the odification into real EFUSE area, you must press “U Efuse” button.

- Save map file in your MP package directory like “C:\Program Files\REALTEK Semiconductor Corp\REALTEK RTL8723AE WLAN NIC Massproduction Kit”

- Press “W Hw Map” button. Write map file into shadow of MP driver.

- Press “U Efuse”. Force MP driver to compare initial shadow and current map file and write into real efuse area.

# HW Setting Map and RF Info

  - MAP: 8723AE use EFUSE or EEPROM 93c46/93c56.

  - RF type and Autoload Status: We support RF type from 1T1R(1*1) for 8723A series. And if NIC is initialized successfully, it display PG, otherwise it displays NPG.

# TX Result Display Block

- Test & Result Display Block

In the block, we provide five major messages now. They are explained as below.

- Result Message: When you press any action button on MP UI, the result message will be displayed here. For example, if you press [REG read] button and the column must display Read Registry OK. If you press [TX DEST Set] button, it must display Set DEST MAC Address OK.

- TX Packets: When you start to do “Packets TX” test, the transmitted packet number of MP is displayed in the column.

- RX Packets OK: When you start to do ‘Packet Rx’ test, the received packet number is displayed in the column.

- RX Packets ERR: When you start to do ‘Packet Rx’ test, the received packet number (CRC fail) is displayed in the column.

# Other

- LED ON/OFF Button

The Led buttons is only for USB adapter. You can press these buttons and let LED on/off.